PROTOCOLS

How the data link layer can combine framing, flow control, and error control to achieve the delivery of data from one node to another. The protocols are normally implemented in software by using one of the common programming languages.  The protocol application is mainly focused on two type of communication channels, noiseless (error free) channels and noisy (error creating) channels. In real life noiseless channels are very less in number or it is not available for communication. All most all channels are noisy channels.  But we must implement protocols for theses two channels.

Noiseless Channels

Let us first assume we have an ideal channel in which no frames are lost, duplicated, or corrupted. We introduce two protocols for this type of channel. They are Simplest Protocol and Stop-and-Wait Protocol

Simplest Protocol

First assume that the channel is noiseless channel, so no need for error control and flow control mechanism. The data delivery is guaranteed, so need of ACK frame. Because of that reason the data traffic is unidirectional. We assume that the receiver can process the data within a small time, so there is no delay for processing.

Design of the protocol

There is no need for flow control in this scheme. The data link layer at the sender site gets data from its network layer, when the network layer has a packet to send. This notification from the network layer is called an ‘event’ in the protocol. The data link layer cannot send a frame until the network layer generates an ‘event’. When the events happened the data link layer gets the data, makes a frame out of the data, and sends it.  The data link layer at the receiver site receives a frame from its physical layer, extracts the data from the frame, and delivers the data to its network layer.

The receiver site cannot deliver a data packet to its network layer until a frame arrives. If the protocol is implemented as a procedure, we need to ‘events’ in the protocol. The procedure at the sender site is constantly running; but there is no action until there is a request from the network layer. The procedure at the receiver site is constantly running; but there is no action until notification from the physical layer arrives. Both procedures are constantly running because they do not know when the corresponding events will occur.

Algorithms- Sender site

Algorithms- Receiver site

Flow diagram

Stop-and-Wait Protocol

If data frames at the receiver site faster than they can be processed, frames must be stored until their use. Normally the receiver does not have enough storage space, so sender must slow down sending, the protocol is stop-and-wait protocol. Here the sender sends one frame , stops until it receives confirmation from the receiver (okay to go ahead) , and then sends the next frame.

Design of Stop-and-Wait Protocol

In Stop-and-Wait Protocol the data traffic is bi-directional. Here we assume that the channel is noiseless, but we need flow control mechanism. Because of the channel is noiseless , no need for error control mechanism.  Here at the sender site, two events can occur, request from the network layer or an arrival notification from the physical layer.  At the sender site after a frame is sent, the data link layer must wait for the acknowledgement from the physical layer about the send frame. The data link layer can’t send frame before getting an acknowledgement.

At the receiver site after the data frame arrives, the receiver sends an ACK (acknowledgement) frame to the sender to send next frame. To perform the receiver site procedure the data link layer at the receiver site must get a data arrival notification from its physical layer. Then the data link layer performs its functions.

Algorithm- sender site

Algorithm- receiver site

Flow Diagram

Noisy Channels

Most of the real life channels are noisy channels. While using these channels we must have flow control and error control mechanism. There are three protocols used to manage noisy channel communication. They are, Stop-and-Wait Automatic Repeat Request (Stop-and-Wait ARQ) ,  Go-Back-N Automatic Repeat Request (Go-Back- N ARQ), and Selective Repeat Automatic Repeat Request (Selective Repeat ARQ) .

Stop-and-Wait Automatic Repeat Request (Stop-and-Wait ARQ)

This protocol adds a simple error control mechanism to the stop-and-wait protocol.  To detect and correct corrupted frames we need to add redundancy bits. The corrupted and lost frames need to be resend in this protocol.  To resend the protocol keeps a copy of the frame. The protocol works based on ACK by the sender and by the receiver. At the time of sending, it keeps a copy of the frame, and starts a timer. If the timer expires and there is no ACK for the sent frame, the frame is resent, the copy is held, and the timer is restarted.

Sequence Number

The protocol specifies that frames need to be numbered. This is done by using sequence numbers. Each sending frame has a sequence number. A field is added to the data frame to hold the sequence number of that frame. For this protocol we need only two sequence numbers, because the ACK is always based on the number of next expected frames number. Assume that we have used ‘X’ as a sequence number, we only need to use ‘x+1’ as the next, there is no need to use x+2, x+3 …..

While sending frames three things can happen

- The frame arrives safely without any problem, and the receiver sends an ACK for the next frame, and so on….

- The frames with a sequence number ‘x’ arrive and destination safely, the receiver sends an ACK, but the ACK is corrupted or lost. In this case the sender resends the frame after the time out, it create a duplicate frame ‘x’. But the receiver can recognize this fact because it expects frame x+1 but frame x was received.

- The frame is corrupted or never arrives at the receiver site; the sender resends the frame, numbered x, after the time out.

We can see that there is a need for sequence numbers x and x+1 because the receiver needs to distinguish between case 1 and case 2. But there is no need for the frame to be numbered x+2 .This means that we can use the sequence number 0, 1,0,1,0, and so on. This is modulo-2 arithmetic. That is in Stop-and-Wait ARQ sequence numbers are based on modulo-2 arithmetic.

Acknowledgement Numbers

Both ACK and data frames must be numbered. The ACK number always announces the sequence number of the next expected frame by the receiver. For example, if frame 0 has arrived safely, the receiver sends an ACK frame with ACK 1 , meaning that frame 1 is expected next. If frame 1 has arrived safely, the receiver sends an ACK 0, meaning that frame 0 is expected next.

Design of Stop-and-Wait ARQ

At the sender site there are three events, request from the network layer, notification from the physical layer and time-out event. The sender site has a variable Sn, which specifies the next frame to send. .  At the sender site after a frame is sent, the data link layer must wait for the acknowledgement from the physical layer about the send frame. The data link layer can’t send frame before getting an acknowledgement. If the physical layer fails to get an ACK within the time limit, the sender resends the frame and starts timer. Then it creates a duplicate copy at the receiver site. By analysing the sequence number the receiver can distinguish the duplicate copy and simply discard it and acknowledge for the next expected frame.  If the frame is lost during the transmission, after the time out the sender resend the frame and starts the timer.

At the receiver site there is only one event, notification from physical layer, it informs the receiver to the arrival of the new frame. Here the protocol has one variable Rn, represents the sequence number of the next frame to receive. If it is duplicate one simply discard it and sends an ACK for the next expected frame. In other case the frame is received and delivered to the network layer and generate an ACK for the next frame.

Flow Diagram

Pipelining

In networking and in other areas, a task is often begun before the previous task has ended, this is known as pipelining. There is no pipelining in Stop-and-Wait ARQ because we need to wait for a frame to reach the destination and be acknowledged before the next frame can be sent. Pipelining improves the efficiency of transmission.

Go-Back-N Automatic Repeat Request ( Go-Back- N ARQ)

To improve the efficiency of transmission (filling the pipe), multiple frames must be in transmission while waiting for acknowledgement. That means in this protocol we can send several frames before receiving ACK; we keep a copy of theses frames until the ACK arrive.

Sequence Number

Frames from the sending stations are numbered sequentially.  If the header of the frames allows ‘m’ bits sequence numbers ranges from 0 to 2m – 1. For example, if m is 4, the sequence numbers are 0 to 14.

Sliding Window

In this protocol the sliding window is an abstract concept that defines the range of sequence numbers that is the concern of the sender and the receiver. Here we have send sliding window and the receive sliding window. The range which is the concern of the sender is called send sliding window. The range which is the concern of the receiver is called receive sliding window. The send window is an imaginary box covering the sequence numbers of the data frames  which can be in transit.

The window at any time divides into four regions.

The  first region  from the left wall of the window defines the sequence  numbers belonging to frames that are already acknowledged. The sender does not worry about these frames and keeps no copies of them. The second region, the second from the left, defines the  range of sequence numbers belonging to the frames that are sent and have an unknown status. The sender needs to wait to find out if these frames have been received or were lost, they are called outstanding frames. The third region, the third from the left, defines the range of sequence numbers for frames that can be sent; however, the corresponding data packets have not yet been received from the network layer. Finally the fourth region defines sequence numbers that cannot be used until the window slides. The window itself is an abstraction, three variables defines its size and location at any time. These are Sf-first outstanding frame,  Sn- next frame to sent, Ssize- size if the send window.

The send window can slide one or more slots to the right when an acknowledgement arrives from the other end

The acknowledgement in this protocol are cumulative, meaning that more than one frame can be acknowledged by an ACK frame. In the above figure, the frames 0,1, 2 are acknowledged , so the window has slid to the right three slots.

The size of the receive window is always 1. The receive window make sure that the correct data frames are received and that the correct acknowledgements are sent. The receiver is always looking for the arrival of a specific frame. Any frame arriving out of order is discarded and need to resent.

The receive window need only one variable Rn( next frame expected) . the sequence numbers of the left of the window belong to the frames already received and acknowledged. The sequence numbers to the right of the window defines the frames that cannot be received. Any received frames with a sequence number in this two regions is discarded. Only a frames with a sequence number matching the value of  Rn is accepted and acknowledged.

The receiver window also slides, but only one slot at a time. When a correct frame is received, the window slides.

Acknowledgement and Resending

The receiver sends an ACK if the frames have arrived safely. If the frame is damaged or received out of order, the receiver is silent and will discard all subsequent frames until it receives the one it is expecting. The silence of the receiver causes the timer of the unacknowledged frames at the sender site to expire. So the sender to goes back and resend all the frames, beginning with the one with the expired timer. This is why the protocol is called Go-Back-N ARQ.  The receiver does not have to acknowledge each frame received. It can send one cumulative acknowledgement for several frames.

Design of  Go-Back-N ARQ

Go-Back-N -ARQ  Vs  Stop and Wait ARQ

Stop and Wait ARQ is a protocol is actually a Go-Back-N –ARQ in which there are only two sequence numbers and the window size is 1.

Selective Repeat Automatic Repeat Request (  Selective Repeat ARQ)

Go-Back N ARQ simplifies the process at the receiver site. The receiver keeps track of only one variable, and no need to buffer out-of-order frames; they are simply discarded, which means resending multiple frames. This is an another mechanism that does not resend N frames when just one frame is damaged; only the damaged frame is resent. This mechanism is called Selective repeat ARQ.

Windows

Selective Repeat protocol uses two windows; send window and receive window. Compared to Bo-Back-N ARQ the send window will be  small in size and the size of the receive window and send window are same. For example if m=4 the sequence number of the window will ranges from 0 to 7 here.

Send window for Selective repeat ARQ

Like Go-Back-N ARQ the send window has four regions. The size of the send window and the receive window are same. In the receiver site the Selective Repeat protocol allows many frames to keep in the window until there is a set in-order frames to deliver to the network layer. Here the receiver never delivers packets out of order to the network layer.

Receive window for Selective repeat ARQ

Those slots inside the window that are coloured defines the frames that have arrived out of order and are waiting for their neighbours to arrive before delivery o f the network layer.

Design

The handling of request event is similar to the previous protocol. The arrival event is more complicated here. An ACK or a NAK (negative ACK) frame may arrive. If a particular frame received out of order , then the receiver sends an  NAK to the sender by specifying the expected sequence number of the frame. Then the sender resends only that frame. If a valid frame arrives the receiver sends an ACK to the sender. Like other protocol here also if the timer expires the sender resends the frame.

At the receiver site the receiver can identify the duplicate frame by using the sequence number. If the frame arrives safely in order then the protocol deliver the frames to the network layer. If the frames arrived out of order then the protocol does not deliver the frames. After deliver the data to network layer the receiver window can slide over to the next group of frames.

Piggybacking

In real life frames are travel in both directions, from node A to B and from node B to A.  A technique called piggybacking is used to improve the efficiency of bidirectional protocols. When frame is carrying data from A to B, it can also carry control information about arrived or lost frames from B. When frame is carrying data from B to A, it can also carry control information about arrived or lost frames from A. When using piggybacking, both sites must use the same algorithm, because it needs to combine two arrival events into one.